Digital System

Assignment 4

Weihao Li

Student number:24210642

(a) Implementing and verifying the standard example programs provided in the lecture "5.2 Simple beag programming examples."

I selected Programming Example 3 (The Asterisk Loop),printing 5 asterisks  to the screen:

Load the Display Address (0x4000) into register r1.

Load the Loop Counter (5) into register r2.

Load the ASCII Character for asterisk (* or 0x2A) into register r3.

Load the Decrement Value (1) into register r4.

Please see more from the file.

(b) A new Buzzer peripheral for the Beag computer. I analysed the buzzer_toy.circ circuit. So I replaced the TTY component with a 16-bit Parallel Register containing input,storage,output.

Data path

Input: The D input of the register is connected to the CPU's main Data In bus.

Synchronization: The register's Clock input is connected to the main System Clock.

Output: The Q output of the register connects to the Frequency input of the Buzzer.

Signal Adaptation: A Bit Extender was placed between the Register (16-bit) and the Buzzer to resolve bit-width compatibility issues.

Please see more from the file

(c)The Simplified Logic (Lecture Style):

Init: Set up registers (lli/lhi).

Load: Use lw to get data from memory

Store: Use sw to send data to the buzzer

Branch: Use bz and bnz for the loops.

1. Setup Buzzer Address

lli r1, 0x03 lhi r1, 0x40

2. Setup Melody Pointer

lli r2, 32

3. Load Note from ROM

lw r3, (r2)

lli r6, -96

lhi r6, 0

sub r7, r6, r2

bz r7, RESET

5. Play Note (Write to Buzzer)

sw r3, (r1)

(d)Please see more from the file

I did this in two steps. First, I changed every line of the assembly program into a 4-digit hex number using the Beag instruction list. For example, the instruction lli uses the number 9, and sw uses the number 5. Second, I changed the music notes from the melody.xlsx file into 16-bit frequency numbers. Then, I put these music numbers into the ROM right after the program code.

The music data starts at address 0x0020. I fixed the data from the Excel file before using it. I changed standard notes like D6 into their hex frequency 0497. I changed rest notes marked as . into 0000 for silence. Also, I fixed the error for the note F#5 so it uses the correct value 02E4.